Procedure for in-flight gain tuning of multi-variable controller in Px4

Longitudinal motion:

States: α, q, Va, γ	Actuators: η, δ 	      Control matrix: Kα,ηKq,ηKα,δKq,δKV,ηKγ,ηKV,δKγ,δKVi,ηKγi,ηKVi,δKγi,δ

- Most important quantity to be controlled is the inclination angle γ. This quantity is typically controlled by a PID controller with Kq,η, Kγ,η and Kγi,η, although a PD approach could also be sufficient at first. This controller will improve the behavior of the short period motion.

- Velocity should be tentatively controlled by thrust commanding at first with KV,δ, KVi,δ. The integrator part is necessary due to model inaccuracies. Gain tuning for a commanded velocity is much less critical as too quick/sluggish control behavior does not jeopardize the aircraft.

- Kγ,δ is to be tuned later and should result in better climbing behavior and a faster phygoid motion.

- Kα,η can be used to alter the short period poles. Due to a small bandwidth, it is very hard to find a satisfying setting during flight, however.

Maja comparison values: K=-155-15000-12024-400-2880

Lateral motion:

States: r, β, p, ϕ 	Actuators: ξ, ζ 		Control matrix: Kr,ξKβ,ξKr,ζKβ,ζKp,ξKϕ,ξKp,ζKϕ,ζKϕi,ξKϕi,ζ

- Most important quantity to be controlled is the roll angle and thus the roll motion. A P controller with Kϕ,ξ should suffice to stabilize the roll axis, however one could add  Kϕi,ξ and Kp,ξ for a PID controller.

- Kβ,ζ can be used to control the side-slip angle and extended with Kr,ζ as a differential addition.

- The dutch-roll motion is counteracted by the aforementioned β-controller. The control behavior can potentially be improved afterwards by Kp,ζ.

- A potential Kβi,ζ should only be applied if information on aerodynamic angles is available via a five hole probe.

Maja comparison values: K=00-8.5-22.530806010